Agency Name: Building Codes Council - Labor, Licensing and Regulation

Statutory Authority: 6-9-40 and 40-1-70

Document Number: 4867

Proposed in State Register Volume and Issue: 42/11

House Committee: Regulations and Administrative Procedures Committee

Senate Committee: Labor, Commerce and Industry Committee

120 Day Review Expiration Date for Automatic Approval: 05/08/2019

Final in State Register Volume and Issue: 43/5

Status: Final

Subject: National Electrical Code

History: 4867

By	Date	Action Description	Jt. Res. No.	Expiration Date

-	11/23/2018	Proposed Reg Published in SR

-	01/08/2019	Received by Lt. Gov & Speaker		05/08/2019

H	01/08/2019	Referred to Committee

S	01/08/2019	Referred to Committee

H	03/28/2019	Resolution Introduced to Approve	4358

-	05/08/2019	Approved by: Expiration Date

-	05/24/2019	Effective Date unless otherwise

provided for in the Regulation

Document No. 4867

DEPARTMENT OF LABOR, LICENSING AND REGULATION

BUILDING CODES COUNCIL

CHAPTER 8

Statutory Authority: 1976 Code Sections 6-9-40 and 40-1-70

8-1100. National Electrical Code.

Synopsis:

The South Carolina Building Codes Council proposes to amend Chapter 8, Article 11, of the Code of Regulations regarding the National Electrical Code.

A Notice of Drafting was published in the State Register on October 26, 2018.

Instructions:

Replace regulation as shown below. All other items and sections remain unchanged.

Text:

ARTICLE 11

National Electrical Code

2017 International Electrical Code Modification Summary

8-1100. National Electrical Code.

NOTE-This article is based upon the National Electrical Code, 2017 Edition, in accordance with the statutory amendments to acts governing the Building Codes Council, except for the modifications referenced below.

This code is identical to the 2017 Edition of the National Electrical Code except for the following modifications:

8-1101. NEC Article 90.2(B)(5) Scope.

b. Are located in legally established easements, rights-of-way, or by other agreements either designated by or recognized by public service commissions, utility commissions, or other regulatory agencies having jurisdiction for such installations, or

8-1102. Repealed.

Fiscal Impact Statement:

There will be no cost incurred by the State or any of its political subdivisions for these regulations.

Statement of Rationale:

The updated regulations will reflect modifications made to the 2017 National Electrical Code, as adopted by the South Carolina Building Codes Council