AGY:	Department of Health and Environmental Control

FII:	2

FIV:	29

PRD:	20031024

EFD:	20050225

EXD:	20050130

REG:	2873

PRI:	10

PRV:	27

COM:	Senate Medical Affairs

Agriculture, Natural Resources and Environmental Affairs Committee 20 HANR

RES:	354

STA:	Final

AUT:	48-1-10

SUB:	Air Pollution, Definitions and General Requirements

HST:	2873

BY	DATE	ACTION DESCRIPTION	COM	VOL/ISSUE	EXP DATE	R. NUM

___	________	__________________________	_______	___________	__________	_______

-	20031024	Proposed Reg Published in SR		27/10

-	20040224	Received by Lt. Gov & Speaker			20040623

H	20040225	Referred to Committee 	HANR 20

S	20040225	Referred to Committee 	SMA

-	20040603	Revised Review Period Exp Dat			20050130

S	20050126	Resolution Intro to Approv	SMA 13			S354

-	20050130	Approved by:  Expiration Date		29/2

TXT:

Document No. 2873

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

Chapter 61

Statutory Authority: S.C. Code Sections 48-1-10 et seq.

R.61-62.1, Definitions and General Requirements

Synopsis:

The United States Environmental Protection Agency (EPA) promulgated a final rule referred to as the Consolidated Emissions Reporting Rule (CERR) in the Federal Register on June 10, 2002 [67 FR 39602].  Pursuant to its authority under section 110 of Title I of the Clean Air Act (CAA), EPA has long required State Implementation Plans (SIPs) to provide for the submission by states to EPA of emission inventories containing information regarding the emissions of criteria pollutants and their precursors.

The purpose of the CERR is to simplify emissions reporting, establish new reporting requirements for PM2.5 (fine particulate matter) and NH3 (ammonia), and establish new requirements for the statewide reporting of area source and mobile source emissions.

R.61-62.1, Definitions And General Requirements, currently requires all facilities that are required to obtain a Title V permit from the Department to submit an emissions inventory every two years.  The Department is revising these requirements in an effort to streamline the emissions inventory reporting process and to be consistent with the CERR.  Accordingly, approximately 50 of the 354 current Title V sources (referred to as Type A sources) will be required to increase their emissions inventory reporting to an annual basis.  However, the vast majority of new and existing Title V sources will realize a decrease in their reporting burden. The CERR requires approximately 80 of the 354 current Title V sources with fewer emissions (referred to as Type B sources) to report their emissions inventory every three years. This will reduce the reporting burden for these sources from every other year to every third year.  For the remaining Title V sources, except those that emit significant hazardous air pollutants (HAPs), there will be an even greater decrease in the reporting burden.  If these sources have submitted an initial inventory, no further reporting will be required.   Those sources that emit significant HAPs will also realize a decrease in their reporting burden.  Instead of submitting inventories every other year, they will be required to submit a summary of their HAP emissions every three years.

The Department has amended R.61-62.1, Definitions and General Requirements, to make the necessary revisions to be consistent with the new Federal emissions reporting requirements and to revise existing State specific requirements to streamline the reporting process.

Discussion of Revisions:

SECTION CITATION:			EXPLANATION OF CHANGE

Section III	The entire section has been revised to incorporate the requirements of the CERR and to streamline the existing regulation.

Instructions:  Replace R.61-62.1 with this amendment.

Text of amendment:

R.61-62.1, Section III has been amended in its entirety to read:

SOUTH CAROLINA

DEPARTMENT OF HEALTH AND ENVIRONMENTAL CONTROL

AIR POLLUTION CONTROL REGULATIONS AND STANDARDS

REGULATION NUMBER 62.1

DEFINITIONS AND GENERAL REQUIREMENTS

SECTION III - EMISSIONS INVENTORY

A. General

Emissions inventory is a study or compilation of pollutant emissions. The purposes of emissions inventories are to locate air pollution sources, to define the type and size of sources, to define the type and amount of emissions from each source, to determine pollutant frequency and duration, to determine the relative contributions to air pollution from classes of sources and of individual sources, to provide a basis for air permit fees, and to determine the adequacy of regulations and standards.

B. Applicability

The provisions of this Section shall apply to all stationary sources:

1.  that are required to obtain a Title V permit as issued by the Department; or,

2.  that are located in a non-attainment area and required to have an air quality permit; or,

3.  that are major sources of hazardous air pollutants (HAP) as defined by Section 112(b) of the Clean Air Act Amendments of 1990 (42 U.S.C. 7412(b)(1)):

a.  whose potential emissions of a single HAP is equal to or greater than 10 tpy (tons per year); or,

b.  whose potential emissions of a combination of HAPs is equal to or greater than 25 tpy.

C. Emissions Inventory Reporting Requirements

1. Reporting frequencies for Type A, Type B, and Non-Attainment Area (NAA) Sources

a. Type A Sources - Title V sources with actual annual emissions greater than or equal to any of the emission thresholds listed for Type A Sources in Table 1 of this Section.  Beginning with the effective date of this regulation, these sources will submit an emissions inventory with the exception of toxic air pollutants (TAP), as defined in S.C. Regulation 61-62.5 Standard 8, Toxic Air Pollutants, and HAP data by March 31 of every year for the previous calendar year.  Beginning in 2006, these sources will submit TAP and HAP data with their annual emissions inventory every third year for the previous calendar year (the three year cycle for HAP and TAP emissions inventories is a March 31, 2006 submittal for 2005 data, a March 31, 2009 submittal for 2008 data, a March 31, 2012 submittal for 2011 data, etc.).

b. Type B Sources - Title V sources with actual annual emissions during any year of the three year cycle greater than or equal to any of the emission thresholds listed for Type B Sources in Table 1 of this Section.  Beginning in 2006, these sources will submit emissions inventories every 3 years for the previous calendar year (The three year cycle for emissions inventories is a March 31, 2006 submittal for 2005 data, a March 31, 2009 submittal for 2008 data, a March 31, 2012 submittal for 2011 data, etc.).

c. NAA Sources - Sources located in a non-attainment area with actual annual emissions during any year of the three year cycle greater than or equal to any of the emission thresholds listed for NAA Sources in Table 1 of this Section.  Beginning in 2006, these sources that are not also Type A Sources will submit emissions inventories every 3 years for the previous calendar year (The three year cycle for emissions inventories is a March 31, 2006 submittal for 2005 data, a March 31, 2009 submittal for 2008 data, a March 31, 2012 submittal for 2011 data, etc.).

| Table 1 - Minimum Point Source Reporting Thresholds by Pollutant (tpy1) |  |  |  |
| --- | --- | --- | --- |
| Pollutant | Annual cycle | Three-year cycle |  |
|  | Type A Sources2 | Type B Sources2 | NAA3 Sources |
| SOx | ³2500 | ³100 | ³100 |
| VOC | ³250 | ³100 | ³100 (moderate O3 NAA ) |
|  |  |  | ³50 (serious O3 NAA) |
|  |  |  | ³25 (severe O3 NAA) |
|  |  |  | ³10 (extreme O3 NAA) |
| NOX | ³2500 | ³100 | ³100 (all O3 NAA) |
| CO | ³2500 | ³1000 | ³100 (all O3 NAA) |
|  |  |  | ³100 (all CO NAA) |
| Pb |  | ³5 | ³5 |
| PM10 | ³250 | ³100 | ³100 (moderate PM10 NAA) |
|  |  |  | ³70 (serious PM10 NAA) |
| PM2.5 | ³250 | ³100 | ³100 |
| NH3 | ³250 | ³100 | ³100 |
| 1 tpy = tons per year of actual emissions. 2 Type A Sources are a subset of the Type B Sources and are the larger emitting sources by pollutant. 3 NAA = Non-Attainment Area. Special point source reporting thresholds apply for certain pollutants by type of non-attainment area. The pollutants by non-attainment area are: Ozone: VOC, NOX, CO; Carbon Monoxide: CO; Particulate matter less than 10 microns: PM10. |  |  |  |

Table 1 is incorporated into this Section as originally published in the Federal Register (67 FR 39602).

2.  Reporting frequencies for all affected sources that do not meet the reporting thresholds listed in Table 1 of this Section.

a. Title V sources that do not meet any of the reporting thresholds for Type B Sources listed in Table 1 of this Section - Beginning with the effective date of this regulation, these sources will review their emissions inventories annually as described in paragraph C(3)(d) of this Section.

b. Sources that hold an air quality permit from the Department and are located in a non-attainment area that do not meet any of the applicable reporting thresholds for NAA Sources listed in Table 1 of this Section - Beginning with the effective date of this regulation, these sources will review their emissions inventories annually as described in paragraph C(3)(d)of this Section.

c.  Major HAP Sources - Beginning in 2006, those Major HAP Sources, as defined by Section 112(b) of the Clean Air Act Amendments of 1990 (42 U.S.C. 7412(b)(1)), who are not already submitting emissions inventories as Type A, Type B or NAA Sources will submit HAP emissions summaries every 3 years for the previous calendar year (The three year cycle for HAP emissions summaries is a 2006 submittal for 2005 data, a 2009 submittal for 2008 data, a 2012 submittal for 2011 data, etc.). HAP emissions summaries shall be submitted to the Department by March 31 following the year of emissions and are not considered emissions inventories.   Information required in a HAP emission summary will include, but is not limited to the following:

i.  A summary sheet showing the source wide emissions of each HAP emitted in excess of 200 lbs/yr;

ii. Calculations for each HAP emitted in excess of 200 lbs/yr source wide.

3. Other Requirements

a. Unless otherwise indicated, all emissions inventories shall be submitted to the Department by March 31 following the year of inventory. All applicable information will be recorded on the current form for reporting emissions data as provided by the Department. Emission estimates will be made for all regulated air pollutants including but not limited to HAPs and TAPs as discussed in the instructions to the current form for reporting emissions data except as provided in paragraph C(1)(a) of this Section.

b. All newly permitted and constructed Title V sources and/or NAA Sources will complete and submit to the Department an initial emissions inventory following the first full calendar year of operation.  These sources shall then submit future emissions inventories on the schedule as described in paragraphs C(1) and C(2) of this Section.

c. Any existing sources that are newly identified as Title V sources and/or NAA Sources will complete and submit to the Department an emissions inventory for the previous calendar year within 90 days of learning of applicability.  These sources shall then submit future emissions inventories on the schedule as described in paragraphs C(1) and C(2) of this Section.

d.  Except Type A Sources, the emissions inventory for all Title V sources and all sources located in a non-attainment area that are required to have an air quality permit shall be reviewed by the source annually to ensure that source reporting thresholds found in Table 1 of this Section have not been exceeded.  A source shall complete and submit to the Department an emissions inventory for the previous calendar year, if as a result of this review it determines that,

i. it has changed from a Type B or NAA Source to a Type A Source, or

ii. its past emissions were less than Type B or NAA Source reporting thresholds and has increased to Type A, Type B, or NAA Source reporting thresholds. These sources shall then submit future emissions inventories on the schedule as described in paragraphs C(1) and C(2) of this Section.

e.  Submittal of emissions inventories outside of the schedules in this Section will be accepted and reviewed only if a modification has occurred that required issuance of an air quality permit since the last emissions inventory submittal by the source.  This modification must alter the quantity or character of the sources emissions.  These sources may submit a new emissions inventory following the first full calendar year of operation after the modification.  These sources shall then submit future emissions inventories on the schedule described in paragraphs C(1) and C(2) of this Section.

f.  Information required in an emissions inventory submittal to the Department will include, but is not limited to, the following:

i.  Information on fuel burning equipment;

ii.  Types and quantities of fuel used;

iii.  Fuel analysis;

iv.  Exhaust parameters;

v.  Control equipment information;

vi.  Raw process materials and quantities used;

vii.  Design, normal, and actual process rates;

viii.  Hours of operation;

ix.  Significant emission generating points or processes as discussed on the current form for reporting emissions data as provided by the Department;

x.  Any desired information listed in 40 CFR 51, Subpart A (June 10, 2002) that is requested by the Department.

g.  A source may submit a written request to the Department for approval of an alternate method for estimating emissions outside of those methods prescribed by the Department.  Such requests will be reviewed by the Department's emissions inventory staff on a case-by-case basis to determine if the alternate method better characterizes actual emissions for the reporting period than the Department's prescribed methods.

h.  Emission estimates from insignificant activities listed on a source’s permit shall be required only in the initial emissions inventory submitted by the source.  If emissions from these insignificant activities have not been included in a past emissions inventory submitted to the Department, the source shall include these emissions in their next required emissions inventory submittal.

i.  The requirements of this Section notwithstanding, an emissions inventory may be required from any source at any time.  Copies of all records and reports relating to emissions inventories as required in this Section shall be retained by the owner/operator at the source for a minimum of five years.

Fiscal Impact Statement:

There will be no increased costs to the State or its political subdivisions as a result of this amendment.

Statement of Need and Reasonableness:

This statement of need and reasonableness was determined by staff analysis pursuant to S.C. Code Section 1-23-115(C)(1)-(3) and (9)-(11).

DESCRIPTION OF REGULATION:

Purpose: This amendment will amend R.61-62.1, Definitions and General Requirements, to make the necessary revisions to be consistent with the new Federal emissions reporting requirements.  In addition, the Department is proposing to revise existing State specific requirements to streamline the reporting process.

Legal Authority: The legal authority for R.61-62 is Sections 48-1-10 et seq., S.C. Code of Laws.

Plan for Implementation: This amendment will take effect upon approval by the General Assembly and publication in the State Register. This amendment will be implemented by providing the regulated community with copies of the regulation.

DETERMINATION OF NEED AND REASONABLENESS OF THE REGULATION BASED ON ALL FACTORS HEREIN AND EXPECTED BENEFITS:

The United States Environmental Protection Agency (EPA) promulgated a final rule referred to as the Consolidated Emissions Reporting Rule (CERR) in the Federal Register on June 10, 2002 [67 FR 39602].  Pursuant to its authority under section 110 of Title I of the Clean Air Act (CAA), EPA has long required State Implementation Plans (SIPs) to provide for the submission by states to EPA of emission inventories containing information regarding the emissions of criteria pollutants and their precursors.

The purpose of the CERR is to simplify emissions reporting, establish new reporting requirements for PM2.5 (fine particulate matter) and NH3 (ammonia), and establish new requirements for the statewide reporting of area source and mobile source emissions.

R.61-62.1, Definitions And General Requirements, currently requires all facilities that are required to obtain a Title V permit from the Department to submit an emissions inventory every two years.  The Department is revising these requirements in an effort to streamline the emissions inventory reporting process and to be consistent with the CERR.  Accordingly, approximately 50 of the 354 current Title V sources (referred to as Type A sources) will be required to change their emissions inventory reporting to an annual basis.  However, the vast majority of new and existing Title V sources will realize a decrease in their reporting burden. The CERR requires approximately 80 of the 354 current Title V sources with fewer emissions (referred to as Type B sources) to report their emissions inventory every three years. This will reduce the reporting burden for these sources from every other year to every third year.  For the remaining Title V sources, except those that emit significant hazardous air pollutants (HAPs), there will be an even greater decrease in the reporting burden.  If these sources have submitted an initial inventory, no further reporting will be required.   Those sources that emit significant HAPs will also realize a decrease in their reporting burden.  Instead of submitting inventories every other year, they will be required to submit a summary of their HAP emissions every three years.

The Department believes that this approach is necessary and reasonable because staff currently spends a great deal of time processing emissions inventories from smaller sources that have fewer pollutants. This streamlined approach will allow staff to focus their efforts on the larger facilities which emit the greatest amount of criteria pollutants and their precursors.

DETERMINATION OF COSTS AND BENEFITS:

There will be no increased cost to the State or its political subdivisions resulting from this revision. The revision will allow the Department to streamline the emissions reporting process by focusing its efforts on large facilities that have the greatest impact on the inventory. This will result in a more efficient use of staff time and will allow staff more time to work on other emissions inventories like mobile sources. In addition, most of the regulated community will realize a decrease in their reporting burden.

Currently, the Department collects emissions inventories every other year from about 354 sources. The CERR will require sources with the greatest emissions to change the frequency of their reporting from every other year to annually. However, the majority of sources will realize a decrease in their reporting burden. The EPA has provided the estimated costs and benefits in the Federal Register notices that are cited in this regulation.

UNCERTAINTIES OF ESTIMATES:

There are no uncertainties of estimates relative to the costs to the State or its political subdivisions. Refer to the above paragraph for cost estimates for the regulated community.

EFFECT ON ENVIRONMENT AND PUBLIC HEALTH:

By implementing this rule, the Department will be able to more efficiently and effectively track emissions of criteria air pollutants and their precursors from large facilities in South Carolina and, thereby, more effectively protect public health.

DETRIMENTAL EFFECT ON THE ENVIRONMENT AND PUBLIC HEALTH IF THE REGULATIONS ARE NOT IMPLEMENTED: none.

Statement of Rationale:

This revision will streamline the emissions reporting process by enabling Department staff to make better use of their time.  No new scientific studies or information precipitated the development of the revision. The bulk of this revision is being made to comply with the Federal mandate.  This revision will change the reporting requirements for all facilities that are required to obtain a Title V permit issued by the Department.  The Title V sources with the greatest emissions will realize a change in their reporting burden from every other year to reporting on an annual basis. Also, they will be required to report additional pollutants. However, the majority of Title V sources will realize a decrease in their reporting burden. Some will only be required to report their emissions inventories every three years and still others will only be required to report an initial emissions inventory.